# Experiment : Temperature and Humidity Measurement using DHT11 and Arduino Uno

1. Introduction This lab project demonstrates the use of the DHT11 sensor to measure ambient temperature and humidity and display the output on a 16x2 LCD screen using an Arduino Uno. The DHT11 sensor provides digital temperature and humidity readings, which are processed and displayed in real-time.

2. Objective: The objective of this experiment is to measure temperature and humidity using the DHT11 sensor, process the data with an Arduino Uno, and display the readings on a 16x2 LCD screen. This experiment aims to demonstrate the interfacing of a temperature and humidity sensor with an Arduino and effectively visualizing real-time environmental data.

3. Components Used

- Arduino Uno

- DHT11 Temperature and Humidity Sensor

- 16x2 LCD Display

- Potentiometer (for adjusting LCD brightness)

- Connecting Wires

4. Implementation & Circuit Diagram: The DHT11 sensor has three main connections:

- VCC: Connected to 5V of the Arduino

- GND: Connected to GND of the Arduino

- DATA: Connected to Digital Pin 2 of the Arduino

The 16x2 LCD is connected to the Arduino using the following pin configuration:

- RS: Pin 0

- E: Pin 1

- D4, D5, D6, D7: Pins 8, 9, 10, 11

- VSS & RW: Connected to GND

- VDD: Connected to 5V

- V0: Connected to the middle pin of the potentiometer for contrast adjustment

5. Code Implementation The following Arduino code reads the temperature from the LM35 sensor, converts the analog reading into a Celsius value, and displays it on the 16x2 LCD screen:

#include <DHT.h>

#include <LiquidCrystal.h>

LiquidCrystal lcd(0, 1, 8, 9, 10, 11);

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

lcd.begin(16, 2);

lcd.clear();

dht.begin();

}

void loop() {

float humidity = dht.readHumidity();

float temperature = dht.readTemperature();

lcd.setCursor(0, 0);

lcd.print("Humidity: ");

lcd.print(humidity);

lcd.print("%");

lcd.setCursor(0, 1);

lcd.print("Temp: ");

lcd.print(temperature);

lcd.print((char)223);

lcd.print("C");

delay(2000);

}

6. Working Principle

- The DHT11 sensor provides digital temperature and humidity readings.

- The Arduino reads the data using the dht.readHumidity() and dht.readTemperature() functions.

- The values are then displayed on the LCD screen, formatted to show percentage humidity and temperature in degrees Celsius.

- The LCD updates the values every two seconds (delay(2000)).

7. Observations

- The LCD correctly displays both temperature and humidity in real-time.

- The serial monitor (if used) can be used to verify readings.

- The potentiometer effectively adjusts the LCD brightness for better readability.

- The sensor provides stable and consistent readings within its accuracy range.

8. Conclusion This project successfully demonstrates how to use the DHT11 temperature and humidity sensor with an Arduino Uno and a 16x2 LCD display. The system efficiently reads, processes, and displays environmental data, making it useful for various applications such as weather monitoring, indoor climate control, and IoT-based smart systems.